The Coolsat 5000 Platinum FTA receiver provides digital free-to-air satellite reception for the USA and Canadian markets. 

The Coolsat 5000 Platinum adds a new faster processor which will allow you to change channels quicker and enjoy one of the best on-screen TV guides available.  This new model also adds component video output for those with new TV's.


MAIN FEATURES

- Full Onscreen TV Guide (EPG)
- Component Y/Pb/Pr Output
- 2Mb Flash ROM for Fast Channel Changing
- Auto Scan Satellite Search
- Universal Remote 
- S-Video Output 
- Dolby Digital Output 
- Easy-to-use User Interface
- 4000 Channel Programmable
- 2 Year Warranty
- Original Unit NOT a Clone

SPECIFICATIONS 

Tuner and Channel 

Input Connector F-type, IEC 169-24,Female 
Loop Through Out F-type, IEC 169-24,Female 
Component Y/Pb/Pr Output 
Frequency Range 950MHz ~ 2150MHz 
Input Impedance 75? unbalance 
Signal Level -25 to -65dBm 
LNB Power 13.5V/18V, max.400mA 
22 KHZ Tone (22?2)KHz, (0.6?0.2)V 
Diseqc Control V1.0/1.2/USALS Compatible 
Demodulation QPSK 
Input Symbol Rate 2 ~ 45 Ms/s 
FEC Decoder 1/2, 2/3, 3/4, 5/6 and 7/8 with Constraint Length K = 7 

MPEG Transport Stream A/V Decoding 

Transport Stream MPEG-II ISO/IEC 
Profile Level MPEG-II MP@ML 

Input Rate 

Max. 15 MBit/s Video Formats 4:3 LetterBox, 4:3 PanScan Video Resolution 720 x 576, 720 x 480 Audio Decoding MPEG/MusiCam Layer 1 and 2 Audio Mode Stereo/Joint Sampling Rate 32 Khz, 44.1 Khz and 48 Khz 

Main System 

Main Processor STi5518 

Memory 

Flash-ROM: 2MB SDRAM: 8MB EEPROM: 32KB 

A/V In/Out 

Video RCA/Cinch, Video Output (CVBS) Audio L/R RCA/Cinch, Volume and Mute Control (16 bits DAC, Max. 2Vrms 

Data Port 

RS-232C Transfer rate 115 kbps, 9 pin D-sub type 

RF Modulator 

RF Connector F-type, IEC 169-24, Female Frequency CH 3: 61.25 MHz CH 4: 67.25 MHz Output Channel CH 3/CH 4 TV Standard NTSC-M Preset Channel CH3 (or TBD) Software changeable by Menu 

Physical 

Size (W x H x D) 260mm x 54mm x 232mm Net Weight 1.8 Kg Operating Temperature 0 ? C~ +45 ? C Storage Temperature -10 ? C~ +70 ? C 

Front Console Physical User Interface 

3-Keys Channel Up/Down, Standby 4-Digit 7-SEG Channel No. or Time Display 2-LED Standby, Remote 

Power Supply 

Input Voltage AC 90 ~ 250V Type SMPS (Switching Power) Power Consumption Max. 20W Standby Power Max. 8W Protection Separate Internal (Input contains surge protection