Pansat Satellite Receiver 2500A Data Transfer (Receiver to Receiver)

*** Connect the Master & Slave receivers by using RS-232 (Null or Twisted) cable ***

# Step #1- Version Transfer

- Connect the master receiver to the TV

- Press the MENU button

- Go to “USER SETUP” and press “OK”

- Go to “SW TRANSFER” and press “OK”

- Press “CH ” button to start transfer (Once the transfer is completed, the audio in the TV unit will release two popping sounds together with the menu OSD)

*** If the percentage bar does not count, reset both the master & slave receivers and check the cable the cable connections again ***

Step #2 – Factory Setting (Master Default List)

- Connect the slave receiver to the TV

- Press the MENU button.

- Go to “FACTORY SETTING” and press the “OK” button

- Press the following buttons in your remote control to begin transfer:		“Menu” – “i” – “1” – “6” – “6” – “8” (Total of six buttons)

- The OSD will read “PLEASE WAIT” for a few seconds (Once the transfer is completed, the audio in the TV unit will release two popping sounds together with the menu OSD)

*** The new version will be updated in the factory default channel data condition by following these steps ***

## Step #3 – Channel data transfer

- Connect the master receiver back to the TV

- Press the “MENU” button

- Go to “USER SETUP” and press the “OK” button

- Go to “SW TRANSFER” and press the “OK” button (“VER.CODE” will appear)

- Press the “VOL ” button to start transfer

- Press the “CH ” button to start transfer

- Once the transfer is completed, the audio in the TV unit will release two popping sounds together with the menu OSD.