Setting up the Pansat 2700

This will be a brief down & dirty “How to” for the 2700 with files included.

Lets get at it,

Download the attached file 2700a.zip to your PC and unzip to a folder of your choice.

Remove Pansat 2700 from box. Attach to PC via RS232C null modem cable.

From the folder you unzipped files to, start & run the application GTRom Loader v3[1].35 [pansat2700A]

Click “Bin File” and select  the 2700 autoroll bin X-87BL_050315_101_api(N)_

Turn your 2700 on via rear switch. When you see the first green LED light up, Click “Download”

Wait for unit to finish flash, DO NOT INTERUPT!

When receiver reboots and displays “ON” proceed to next step of loading a channel list

NOTE; The channel list provided is working5 by Wingnutt101 for LEGACY lnbs ONLY

To load a channel list on your 2700, start & run the application Ch Editor187 (Pansat 2700A)

When Pansat Channel Editor 2700a appears, click file, open, select the channel list working5-legacy-2700

At the top of the screen click where it says “Tools” and goto “Connect to receiver’ Then click “Download”

Wait till finished and unit will reboot to channel 001. Your Pansat 2700 is now flashed and a Legacy Dishnet channel list has been loaded.

Disconnect receiver from PC and install to dish & TV connections.

Enter “Menu” and goto Antenna setup and make the necessary changes of the DiSEqC settings to match your own configuration.

Exit menu and select Channel 300, The “Downloading Data” should appear on your screen, and after a few minutes (up to 10 minutes) TV should appear.

Enjoy your new receiver 